# SIMULATION MODEL DEVELOPMENT IN SMEs – A CRITIQUE

John Geraghty* and Cathal Heavey**

*Department of Manufacturing and Operations Engineering, University of Limerick, Limerick, Ireland. Tel: +353-61-213192, Fax: +353-61-202913, Email: john.geraghty@ul.ie

**Department of Manufacturing and Operations Engineering, University of Limerick, Limerick, Ireland. Tel: +353-61-202891, Fax: +353-61-202913, Email: cathal.heavey@ul.ie

Abstract:

Manufacturing systems are becoming more automated and, as a result, more complex and difficult to control and operate.  It is important that manufacturing engineers have tools that properly support their activities on a continuous basis.  A number of tools are available to address manufacturing systems design and operations.  Of these tools simulation is the only one capable of dealing with all the detailed decisions that must be made in designing and operating these systems.

This paper describes a simulation project carried out in a SME.  The SME manufacturing system is best described as a batch production flow shop.  The development and validation of the model took approximately nine man-months.  Once validated a wide range of experiments where carried out on the model including, work schedules, improving efficiency of the bottleneck station, capital investment, sequencing and order release mechanism.  Through these experiments the company could see a real need for simulation based tools to support their activities.  To illustrate this point this paper will describe the use of simulation to support a capital investment decision.  However the main purpose of the paper is to highlight practical issues relating to the introduction of simulation into a SME.  These issues mainly relate to simulation software selection and how this software is best utilised.

Keywords: Simulation, SME, and Simulation Software.

# Introduction

The SME in question manufactures consumables for the mining industry.  Its manufacturing system operates as a batch production flow shop and a schematic diagram of the layout of machines and processes within the system is shown in Figure 1.  Batch sizes are determined by the customer orders and therefore, can vary in size from 1 part to 500 or more parts.  The number of different part types that can be produced within the system is very large.  However it is possible to classify parts based upon similarity in design and product routing.  Therefore, there are five basic product classes that are produced within the system, namely Tunnelling rods, Extension rods, Male/Female rods, Tube rods and Taper rods.

Figure 1 Schematic Diagram of the Layout of the Rod Plant

# Capital Investment Decision – An Example Experiment

One option available to management to increase the performance of the system was to invest capital in new machines and equipment.  The furnace area is the main bottleneck within the facility.  Therefore, it was decided that any capital investment should be conducted such that it alleviates the constraint placed on the system by this bottleneck.  The option of investing in a second carburising furnace was investigated by management.  Capital was found to support the purchase of a furnace slightly smaller than the current furnace. The simulation model of the rod plant was utilised to determine if this investment would result in tangible benefits and to determine if a new bottleneck would develop in the system.

## Experiment Design

Two scenarios were considered for the purposes of this experiment.  In the first scenario the system was simulated with the current carburising furnace, in the second scenario the proposed new furnace is added.  An eight-week production schedule from the system was obtained and used to drive the scenarios.  The first two weeks of the production schedule were used as the warm-up period. Statistics were collected at the end of each week for the remaining six weeks.

For the second scenario batch sizes were increased by 30%.  This was to ensure there would be enough parts entering the system to utilise the extra capacity.  The figure of 30% was selected after consultations with management indicated that this would be a reasonable estimate of the amount of new business they could expect to generate if there was extra capacity in the system.  It should be noted that this simplifying assumption does not take into consideration the effects an increase in the number of batches would have on the capacity of the system.  Also as the range in product sizes is large, product mix is an important factor in furnace utilisation.  In order to simply the experimentation both batch sizes and product mix where not considered.

Ten replications of each scenario were conducted.  Statistics were collected for average number of parts carburised, average WIP remaining in system at end of each week, average time in system, as well as machine utilisation figures for the furnaces, the CNC machines, the straightening machines and the friction welder.

## Experiment Results

There was a significant increase in the average number of rods carburised from 1591 per week to 2287 per week.  There was a significant reduction in the amount of WIP remaining in the system at the end of each week down from 2427 to 1671 rods.  Average time spent in the system by parts under scenario 1 was on average 10910 minutes or approximately seven days and fourteen hours.  The average time spent in the system by parts under scenario 2 was 11105 minutes or approximately seven days and seventeen hours.  In Figure 2 a time series of the time spent in the system is shown.  This graph shows that the time in the system increases for scenario 1 and decreases for scenario 2.  The average number of furnace loads achieved under scenario 1 was approximately 16 loads per week.  This compares with an average of 24 loads in total between the two furnaces under scenario 2.  As can be seen from Table 3, the utilisation of all other machines increased under scenario 2.  The most important of these increases are the CNC 350, the Friction Welder and the manual straightener.

Figure 2 Average Time Parts Spend in the System

Table 1 Average Utilisation of the Furnace in Scenario 1

| Week | No of Ops | Blocked | Cycling | Idle |
| --- | --- | --- | --- | --- |
| 1 | 15.00 | 0.00 | 97.03 | 2.95 |
| 2 | 17.00 | 0.00 | 98.67 | 1.49 |
| 3 | 17.30 | 0.00 | 98.59 | 1.64 |
| 4 | 17.20 | 0.00 | 98.18 | 2.15 |
| 5 | 16.30 | 0.00 | 90.94 | 9.28 |
| 6 | 13.10 | 0.00 | 81.33 | 18.92 |
| Average | 15.98 | 0.00 | 94.12 | 6.07 |

Table 2 Combined Average Utilisation of the two Furnaces in Scenario 2

| Week | No of Ops | Blocked | Cycling | Idle |
| --- | --- | --- | --- | --- |
| 1 | 23.80 | 4.00 | 71.90 | 24.32 |
| 2 | 22.40 | 2.82 | 63.01 | 34.43 |
| 3 | 29.60 | 6.92 | 87.71 | 5.47 |
| 4 | 25.60 | 4.77 | 76.77 | 18.54 |
| 5 | 22.10 | 3.34 | 63.87 | 32.80 |
| 6 | 19.90 | 1.80 | 61.68 | 36.79 |
| Average | 23.90 | 3.94 | 70.82 | 25.39 |

Table 3 Average Machine Utilisation

| Machine | Scenario | No. of Ops | Cycling | Setup | Idle | Broken Down |
| --- | --- | --- | --- | --- | --- | --- |
| CNC 302 | 1 | 10.33 | 24.68 | 5.81 | 57.77 | 11.29 |
|  | 2 | 10.97 | 29.52 | 6.73 | 48.17 | 15.58 |
| CNC 350 | 1 | 16.17 | 53.63 | 16.07 | 19.68 | 10.80 |
|  | 2 | 16.77 | 70.26 | 16.77 | 0.23 | 12.88 |
| - CNC 4x4 | 1 | 9.28 | 43.52 | 18.75 | 25.99 | 11.87 |
|  | 2 | 10.45 | 50.61 | 19.16 | 20.53 | 9.73 |
| Welder | 1 | 17.34 | 39.19 | 15.63 | 43.37 | 1.87 |
|  | 2 | 18.92 | 51.92 | 16.87 | 29.26 | 2.04 |
| Manual | 1 | 36.13 | 57.36 | 0.00 | 42.64 | 0.00 |
| Straightener | 2 | 38.55 | 83.41 | 0.00 | 16.59 | 0.00 |
| Eithel | 1 | 8.50 | 27.29 | 0.00 | 72.71 | 0.00 |
| Straightener | 2 | 8.75 | 35.15 | 0.00 | 64.85 | 0.00 |

Table 4 Utilisation of the Manual Straightener in Scenario 2

| Week | No. of Ops | Cycling | Idle |
| --- | --- | --- | --- |
| 1 | 26.70 | 99.50 | 0.50 |
| 2 | 39.40 | 97.16 | 2.84 |
| 3 | 46.00 | 92.01 | 7.99 |
| 4 | 58.50 | 92.85 | 7.15 |
| 5 | 37.50 | 54.19 | 45.81 |
| 6 | 23.20 | 64.76 | 35.24 |
| Average | 38.55 | 83.41 | 16.59 |

## Discussion

The addition of the new carburising furnace would result in tangible benefits to the performance of the system.  It would result in increased number of carburised products per week with a reduced average WIP remaining in the system at the end of each week and minimal increase in average time in system.

In addition, product mix and batch sizes will have a large input into determining the capacity of the system.  This experiment does not consider how either system would cope with changes to these parameters.  If, for instance, the number of tube rods (which have large diameters) or the number of long rods were to increase it would result in a decrease in the number of carburised product per week.  The number of batches entering the system will clearly effect the amount of time the CNC machines spend setting up. Management believes that a maximum of three years should be allowed for returns on capital investments to be achieved.  Discussions with management indicated that, based on the results of this experimental work, they would expect to see a return on investment in two and a half years. However further experiments on product mix and batch sizes would be required to give a more precise value for the return on investment period. also indicates that the furnaces can become blocked at the end of their cycles, an average of 3.94% of available time.  This implies that there was a load in the cooling tower when one of the furnaces attempted to discharge its load to the cooling tower.  If this were to happen in reality then the entire jig would have to be scrapped.  The jig cannot remain longer in the furnace than its carburising cycle requires and cooling of the jig must begin immediately after carburising under controlled conditions.  Therefore, if a second furnace was to be introduced into the furnace area then loading of each furnace would have to be conducted such that it did not attempt to discharge its load while there was still a load in the cooling tower.  This could be achieved by staggering the furnace loads.  From examination of the utilisation of the individual furnaces in scenario 2 (these figures are not shown due to space restrictions) maximum utilisation of approximately 80% would only be achievable when furnace loads are staggered.

As can be seen from Table 1 the furnace under scenario 1 is being utilised very close to the maximum possible.  In fact for the first four weeks when the system is heavily loaded the average utilisation is approximately 98%.  The furnace, therefore, is clearly constraining the throughput of the system.  Under scenario 2, with the addition of a second furnace the combined utilisation of the two furnaces reduces to approximately 75% for the four heavily loaded weeks (see Table 2). The furnace area is no longer the main bottleneck that is constraining the performance of the system.  From Table 3 it is clear that the CNC 350 was utilised to its maximum capacity under scenario 2 as the average idle time per week over the six week period was 0.23%.  The CNC 350, therefore, will become a bottleneck if a second furnace is introduced and the load on the system is increased.  It may be possible to relieve this bottleneck by allowing the CNC 4x4 to work on jobs that were destined for the CNC 350 during its idle time.  Another possibility would be to attempt to reduce cycle times, set-up times and the number and length of breakdowns.

Increasing the size of the production batches by 30% increased the system load for the second scenario.  This had the effect of increasing machine cycle times without substantially increasing the number of set-ups or operation dependent breakdowns.  If, however, the system load was increased by increasing the number of batches released into the system then time spent on set-ups and repairing operation dependent breakdowns would also be increased.  This could result in bottlenecks at other machines and work-centres, for instance the CNC 4x4.

Table 3 also indicates that the manual-straightening machine was utilised an average of 83% of available time per week.  Examination of Table 4 reveals that for the first four-week period when the system was heavily loaded the average utilisation of the manual-straightening machine was 95%.  It can be concluded therefore, that the manual-straightening operation will also be a bottleneck if a second furnace is introduced and the load on the system is increased.

## Experiment Conclusions

In summary the above set of experiments provided the following information to the company:

- The introduction of a second furnace would have tangible benefits for system performance in terms of increased throughput at reduced average WIP while maintaining the average time in system.

- Return on the investment could be achieved within 2 ½ years, however the effect of product mix and batch sizes should be investigated further to ensure that the eventual return on investment would be reasonable.

- The loading of two furnaces would have to be conducted in a controlled manner so as to ensure that there would not be a load in the cooling tower when a furnace would be due to discharge its load.

- If it was desired to operate the two furnaces at more than 75-80% utilisation it would be essential to consider purchasing a second cooling tower.

- If the system load was increased by increasing batch sizes then CNC 350, CNC 4x4 and the manual straightening operation would become the main bottlenecks in the system.

# Simulation Expertise Requirements

The above simulation experiment exercise illustrates the benefits a SME can obtain from the utilisation of simulation.  One objective of the simulation project was to introduce simulation into the organisation so that it could be used on a continuous basis after the completion of the project.  This seemed sensible as the model took nine man-months to complete.  Scrapping the model is an obvious waste of resources, not to mention the potential benefits that could be gained from future use of the model within the organisation.  The model was discarded after the completion of the project as the company lacked the necessary in-house expertise to maintain and use the model.

Expertise required by the company to use the model could be categorised into:

- Knowledge of the software package as well as the programming details of the model,

- Knowledge of the system being modelled, and

- Knowledge of statistics and experimental design procedures to be able to conduct and analyse experiments.

Company personnel have the necessary expertise in (2) and could with training acquire the necessary expertise in (3).  Even so activities in (3) above are poorly supported.  Also execution time for simulation models is a major constraint to experimentation.  To illustrate the model used for the experiments described earlier took approximately 45 minutes for 1 replication of 8 weeks simulated production on a Pentium II with 64 Megs of RAM.  While noting that major work both in research and application development needs to carried out in this area, this is not addressed in this paper.  Issues relating to (1) are discussed in the remainder of this paper.  Noting that the level of expertise required in (1) is dependent on how the simulation model is implemented, we critically review simulation software that is in our view one cause that is hindering the more widespread use of simulation within manufacturing companies and especially SME.

# Simulation Software Implementation Issues

The chosen simulation package (WITNESS) used in the project militated against developing a user-friendly model, which could be used by company personnel.  Problems with the software can be summarised as follows (Geraghty, Heavey & Linnane, 1998):

- WITNESS does not provide separate interfaces for users and developers.  In other words, no facility is provided to hide programming details from the user.  Because of this it is virtually impossible for company personnel to use the model, unless they become expert in the use of the WITNESS simulation package.

- WITNESS has poor interfaces to external data sources.

- WITNESS is not object-oriented.  Object-oriented simulation that allows data encapsulation, inheritance and polymorphism is a virtual necessity for the modelling of complex systems.  In WITNESS, all variables are global variables, making it very difficult to code and maintain the model.

- It is difficult to model complex systems using WITNESS because WITNESS does not provide low level modelling primitives.

Each of the above points is briefly elaborated on in turn.

## Separating the User from the Developer

Figure 3(a) illustrates how WITNESS operates; the user and the developer use the same interface.  A number of recently developed or extended simulation packages (ARENA (Systems Modelling Corporation), Simple++ (ASEOP GmbH), TED (F & H Simulations Incorporated)) operate as in Figure 3(b).  This permits the development of customised user-friendly simulation software.

Figure 3 Separation of Model User from Model Developer

To illustrate screen dumps showing what is involved in changing the `Maximum Cooling Tower Duration’ parameter of the carburising furnaces in WITNESS and SIMPLE++ are given in Figure 4 and Figure 5, respectively.  To change this parameter in WITNESS one must access detailed programming code (see Figure 4), in SIMPLE++ this parameter can be accessed via a customised DialogBox (see Figure 5).

Figure 4 Screen Dump of Witness Function for Selecting Carburising Furnace Cycle Time

Figure 5 Furnace Area Dialogue Box from the Simple++ Model

## Linking Simulation to External Data Sources

If simulation is to be implemented successfully as a desktop resource that will be used on a continuous basis within a company, data input activities must be automated.  Simulation models require large amounts of data, for example, part routings, processing times and production schedules. The simulation model will need to be linked to the production planning system to access data relating to product routings and schedules.  Links to a shop-floor data collection system is required to access shop-floor status data and possibly in highly automated systems to update simulation parameters such as process and failure times and distributions.

## Object Oriented Programming

The main ideas of object oriented programming where first demonstrated in a simulation language called SIMULA in the late 1960.  However, it is only recently that object oriented techniques have become to be applied to manufacturing simulation packages, for example long established packages such as SIMAN (ARENA) and WITNESS (Version 9).  These packages however are not truly object-oriented, as they are legacy packages.  Examples of new products that are truly object oriented are MODSIM III, which could be regarded as a general simulation development toolkit and SIMPLE++, which could be regarded as a general simulation development toolkit for the manufacturing domain.  A more recent entrant into this market is Taylor Enterprise Dynamics (TED) which is based on the object-oriented paradigm.

The advantages that object-oriented programming brings to simulation is similar to those achieved in software development in general.  To illustrate this point further we examine the re-coding required to carry out the experiment previously described.  It took two weeks to write and verify the code to enter a second furnace into the WITNESS model.  The furnace operational logic is encoded in a series of WITNESS functions and, because WITNESS does not distinguish between global and local variables, these functions could not be accessed simultaneously by a second furnace.  The functions and variables required to model the operational logic of the furnace therefore had to be identified and copied.  If the model was developed in an object-oriented based simulation package such as SIMPLE++ this task would have been greatly simplified and would be accomplished within minutes or hours.  In the object-oriented framework the furnace would have been an object.  Due to encapsulation it could be easily integrated with other modelling objects through their respective object interfaces.

## Model Complexity

A number of simulation packages contain pre-defined objects that are used to construct models.  These objects have limited flexibility.  Examples of two such packages is WITNESS and PROMODEL (PROMODEL Corporation).  These packages have the advantage that they have low training requirements and are useful tools if their pre-defined objects met ones modelling requirements.  However very often this is not the case.  While there may be strong similarities in terms of modelling requirements within manufacturing sectors (i.e., assembly), from experience we find that, in general, the range and level of complexity is large among companies.

It can be justifiably asked; is there a need to model detailed aspects of manufacturing systems?  This is impossible to answer in the absolute sense as simulation objectives and systems differ.  However we have found that in most cases there is a need.  Another paper that supports this view is a paper by Davis, Setterdahl, Marco, Izokaitis and Bauman (1993).  They developed a simulation model of a Flexible Manufacturing System (FMS) using the conventional simulation tool SIMAN and numerous FORTRAN patches.  It took two man-years to develop the model, and the final model proved extremely difficult to validate.  They questioned was such complexity of the model required?  They argue yes and give cases to justify their answer.  As an example for the FMS, a proposed vendor of a tool delivery system gave results of a simulation study that projected average process utilisation would exceed 70%.  The detailed model demonstrated that the average process utilisation would be less than 40%.  Another set of experiments on the same model showed huge variation in make span (75% to 200%) depending on what tools resided in the machines at the beginning of each day (all other conditions being equal).  While accepting that simple models are of great benefit to engineers in understanding the main operating issues of their manufacturing systems, where complex manufacturing systems are concerned detailed simulation models are required for detailed design.

Low level modelling primitives would lower the effect required to model complex logic.  Relating this back to the model previously described, the furnace loading activities was a complex activity to model.  In WITNESS a range of “dummy” objects and modelling tricks were required.  Utilising low level modelling primitives would have greatly eased this modelling task.

# Conclusions

This paper described a case study that illustrated the use of simulation within a SME.  While model development took a lot of effort the company were interested, after seeing the benefits of simulation, in developing simulation within the company.  Unfortunately after the simulation project stopped the company could not use the simulation software.  There are many difficulties to overcome in introducing simulation successfully in a company.  This paper highlights problems related to simulation software selection and implementation.  We conclude that if a company wants to develop simulation into a tool that can be used continuously to support manufacturing they should utilise a simulation software package that has the following features:

- Allows the separation of the user and developer interfaces;

- Supports object-orient programming;

- Contains low level programming primitives;

- An open product that can be easily linked to external data sources and external applications.

# REFERENCES

ASEOP GmbH http://www.tecnomatix.com/simple.htm.

Davis, W. J., D. Setterdahl, J. Marco, V. Izokaitis and B. Bauman (1993). Recent Advances in the Modelling, Scheduling and Control of Flexible Automation. In Proceedings of the 1993 Winter Simulation Conference, Los Angeles, CA, USA, IEEE New York, NY, USA.

F & H Simulations Incorporated http://www.taylorii.com/.

Geraghty, J., C. Heavey and J. Linnane (1998). Optimisation of a Batch Production Flow Line with Bottlenecks Using Computer Simulation. In Proceedings of The Fifteenth Conference of the Irish Manufacturing Committee, University of Ulster, Jordanstown, N. Ireland, Academic Publications, UUJ.

PROMODEL Corporation http://www.promodel.com/.

Systems Modelling Corporation http://www.sm.com/.