Supervisory Control Under Incomplete Plant Model

##### Houshang Darabi

Mohsen A. Jafari

Department of Industrial Engineering

Rutgers University

96 Frelinghuysen Rd.

Piscataway, NJ 08854-8018

Houshang@eden.rutgers.edu

Jafari@rci.rutgers.edu

###### Abstract

Ramadge-Wonham (RW) framework for designing a supervisory controller for a given plant and user specification, has been widely addressed by many researchers. The framework accepts a generator (or plant model), G, and a specification (or imposed constraint on the plant), H, and generates the supervisory controller, S. It assumes that plant is known and deterministic. However, in many practical cases the designer is not completely aware of the real generator GR. In this paper the above assumption is removed. It is assumed that G is not fully known. Using the available Gand the user specification H, RW procedure is applied to a supervisor S. Then the family of (real) generators, GR, which can be supervised under S while satisfying the specification His introduced and different cases are discussed. We establish the mathematical framework for one of the cases where an event originally controllable becomes uncontrollable.  It is argued that the complement of GR which violates the specification Hcan be partially synthesized by available recovery and diagnosis techniques in RW literature if the violation is already known. Otherwise the violated specification must be handled in run time.

1.0  INTRODUCTION

Ramadge-Wonham (RW) framework for designing the supervisory control of discrete event systems accepts the plant model and the user specification, and generates the supervisory controller [1,3,4,5,6,7]. The generated controller is maximally permissive, and is live. It guarantees correctness and non-blocking properties, which make the method unique in comparison with the other available approaches in the literature (e.g. Petri Nets and Formal Languages [21]). The initial version of RW method, introduced by Ramadge and Wonham [6,7,4], made the following assumptions: 1) Plant is deterministic, 2) Plant events can be fully observed, 3) The devices used to collect information on the plant events are not subject to failures, 4) Plant must be controlled by a single agent (centralized control assumption) and 5) Plant model is fully known or correct. Assumptions one, two and four have already been relaxed by a number of researchers. Non-deterministic plant models [8,9] have been investigated using different techniques like trajectory models and failure semantics. The relaxation of the second assumption yields another class of supervisory control problems, Supervisory Control under Partial Observation, which has been fairly discussed by many [10,11,12,13,18]. Decentralized supervisory control, introduced by Lin and Wonham [17], has also been discussed in the literature [14,15,16]. However, the relaxation of assumptions three and four has not been addressed yet. In this paper we address the issues around relaxing assumption four. We assume that the supervisor is originally designed under only partial knowledge of the plant behavior.

It should be noted that the discussion here is different from diagnosis of discrete event systems originally addressed by Lin [19,20].  There, a correct and a complete knowledge of the plant is assumed including fault or abnormal states which are the subset of plant states. Therefore, at the time of failures, the controller already has embedded in it the set of appropriate transitions to follow. In our case, such a priori knowledge is not assumed and not embedded in the model. What we are suggesting is that the supervisory controller should be able to adapt itself to unknown events or to failures in the sensory devices in some autonomous manner. Every time that such an event occurs the supervisor must determine if its laws in terms of enabling and disabling of the events in the plant are still appropriate (conforming to control specification) under the new conditions. If not appropriate then the supervisory control laws must be modified. There are always some events which if occur would result in the shut down of the system and can only be resolved by a manual intervention. The main issue is to find the set of events and conditions under which the adaptation can be done within the supervisory controller itself, and outside there must be some manual intervention.

2.0 BACKGROUND REVIEW

Consider the plant generator defined in [7]: G=Q,Σ,δ,q0,Qm where Q is the set of states, Σ is the alphabet (event) set, δshows the transition function such that δ:Σ×Q→Q, q0 is the plant initial state and Qmis the set of marked states where Qm⊆Q. The set of events is divided to two disjoint subsets: Σc the set of controllable events and Σuthe set of uncontrollable events such that: Σ=Σu∪Σc and Σc∩Σu=φ. Let  L=L(G) be the closed language (L=L), which includes all possible strings that can be generated by the plant. Suppose that the specification imposed on the plant divides the plant language to two sets of strings: the legal languageHand the illegal language L−H.  LetS=S,Φ be a complete supervisor obtained by the procedure given from Ramadge and Wonham [6,7] where S=(X,Σ,g,x0,Xm). We define K as the corresponding coupled language i.e. K=SUPC(H)=L(SGc). The following results are given in the literature:

- L,HandKare closed [6,7].

- The coupled language LSGc is closed and non-blocking, proper and non-rejecting [7].

- Suppose s∈K and sσ∈L. Then sσ∉K if and only if there exits a string l∈Σu such that sσl∈L−H[17].

3.0 The Problem of Supervisory Control Under Incomplete Plant Model

Consider the coupled system of figure 1 constructed by RW method (figure 2). In this system supervisor(S) observes the events generated by plant(G) and sends the feedback policy(γ)to the plant. Feedback policy includes enabling and disabling of controllable events. We modify the above supervisor by assuming that Gmay not be correct or complete. In other words, GRthe true plant model is assumed to be unknown at the time of designing the supervisor. There is no guarantee that L(SGc)is a subset of the legal language if GR≠G. Two questions arise here: 1) Does the real coupled system (figure 3) violates H? 2) If it does, how one must modify S in order to keep L(SGRc)inside H? To answer the first question, we define set R:R=Gp:L(SGPc)⊆H.In other words, R is the set of all generators coupled with S such that the supervised plant language is still in H. We refer toR as the set of legal generators. Therefore if GRÎR then no modification of S is required. However if GRÏRthen L(SGR)⊄Hand S should be somewhat revised to obtain a new supervisorSR, such that L(SRGR)⊆H.

A generic supervisory control model for the above problems is represented by figure 4.  The model is different from regular feedback system (figure 1) by a new meta supervisory level. In this level we observe the real plant events, compare them with the originally known plant events (or L(SGc)). If the behavior is different from that of L(SGc) and it does not belong to R, then the meta supervisor computes a new control policy such that the real system remains in the specification H; otherwise, it has no effect on the original closed loop system given by figure 1.

To implement the meta model in the above system, the three elements are required:

- A monitoring mechanism for observing the actual plant and comparing it with the desired one (element number 1 in figure 4). An ideal mechanism for this would be the one that observes any event including events in Σor outside Σ. It should be noted that observing an event which does not belong to the predefined set of events Σ, does not mean a violation from H, because of the self loop property of the specification language [7], unless, the new event is illegal itself. But, the successor strings to the new event may create an illegal behavior. Therefore it is required for element 1 of the meta supervisor to record all events such that the element 3 could use the new strings for necessary revisions.  Such an observation/comparison scheme may require a very complex sensory network in practice.

- Legal behavior verification unit (element number 2 in figure 4): This unit verifies whether the observed behavior is a subset of legal behavior H or not. As mentioned earlier if GR∈R then we know that the generated language is a subset of H. So it is important to derive R. GRcould be different from Gin a number of ways:

- G assumes that an event is controllable but actually it is not, i.e., CU(σ)⇔(σ∈Σc,Gand σ∈Σu.GR),

- G assumes that an event is uncontrollable but actually it is controllable, i.e., UC(σ)⇔(σ∈Σu,Gand σ∈Σc.GR).

### A new event is observed leading to a new state

III.      A new event observed leading to an already known state, etc.

Our general approach to derive R is illustrated by figure 5. The notation RW−1shows that the method attempts to establish all generators, which if coupled with Sgenerate a language in H. We recall that RW method accepts the generator and H, and obtains S (figure 2).

3-Supervisor revision module: In the case of violation from H, it is necessary to modify the current supervisor. We propose two steps for this stage. First the plant should be returned back to a legal state (if it is already in an illegal state). Second the supervisor should be revised and reset such that the violation never happens again. Unfortunately, in practice the violation is not already known. Therefore step one can only be handled in run time. In other words, it would be impossible to formulate a general policy for step one. The reader notices that if the violation is already known then the problem becomes the regular diagnosis problem, which has been addressed [19,20].

  - Computation of R

#### Here we will only discuss this for the case CU(σ). The following lemma obtains the set Rfor this class. The proof is omitted.

Lemma1:K is not affected by CU(σ), if and only if there is no string s∈Ksuch that Φ[g(x0,s)](σ)=0.  In other words CU(σ)does not violate Hif and only if σhas never become disabled by supervisor S

Now what happens if the observer in figure 4 observes σ∈Σc,Gas an uncontrollable event (a typical case might be when the observer observes string sσwhere s∈Kbut sσ∉K). The necessary and sufficient condition discussed in lemma 1 show that any violation from H is originated by at least one string s∈Ksuch that Φ[g(x0,s)](σ)=0. Therefore to obtain the modified supervisor for string s where s∈Kand Φ[g(x0,s)](σ)=0 we do the following: let s=σ1σ2⋯σn (nis finite because Lis regular). Define r=maxi:1≤i≤n∧σi∈Σc. In other words, σr is the last controllable event in the sequence σ1σ2⋯σn (if rdoes not exist then K is null because sσ∈Landsσ∉H). Change the feedback rule from Φ[g(x0,σ1σ2⋯σr−1)](σr)=1to Φ[g(x0,σ1σ2⋯σr−1)](σr)=0. In the supervisor obtained as such, delete any blocking state (this can be done by the procedure introduced by Ramadge and Wonham [6]).

RW DESIGN

Figure 2.  RW Design

Figure 1.  Designed Feedback Loop

Figure 3. Real Feedback Loop

DESIGN

Figure 5.  Inverse Design

New   Supervisor Computation

Yes

No

No

Yes

Figure 4.  Recovery Model

Observer

1

2

3

4.0 CONCLUSION AND FUTURE WORK

Recovery problem, a new framework for RW supervisory control problem while the real plant is unknown, was introduced.  Due to fair mathematical structure of RW method it was possible to develop the theoretical basis of recovery problem. We established the class of non-sensitive generators with respect to change in controllability and uncontrollability of an event. Our research continues on the other cases. Furthermore, we examine cases where only partial observation of events is possible.

## 5.0  REFERENCES

[1] F. Lin, A. F. Vaz and W.M. Wonham, “Supervisor    specification and Synthesis for Discrete Event Systems”, Int. J. Control, vol. 48, no. 1, pp.   321-332, 1988.

[2] L. Yong and W.M. Wonham, “On Supervisory  Control of Real-Time Discrete –Event  Systems”, Proceedings of the American Control  Conference, pp.1715-1720, 1987.

[3] W. M. Wonham and P. J. Ramadge, “Modular  Supervisory Control of Discrete-Event Systems”, Maths. Contr., Signals Syst., vol. 1, no.  1, pp. 13-30, 1988.

[4] P. J. Ramadge and W. M. Wonham, “Modular  Feedback Logic for Discrete Event Systems”,  SIAM  J. Control and Optimization, vol.25, no. 5,   pp.1202-1217, 1987.

[5] W. M. Wonham, “ A Control Theory for Discrete- Event Systems”, NATO ASI Series, Vol.  F47, pp. 1-21, 1988.

[6] W. M. Wonham and P.J. Ramadge, “ On the  Supremal Controllable Sublanguage of a Given  Language”, SIAM  J. Control and Optimization,  vol.25, no.3, pp.637-657, 1987.

[7] P. J. Ramadge and W. M. Wonham, “Supervisory  Control of a Class of Discrete Event  Processes”, SIAM  J. Control and Optimization, vol.25, no.1, pp.206-230, 1987.

[8] A. Overkamp, “Supervisory Control Using Failure  Semantics and Partial Specifications”,  IEEE Trans. Auto. Control, vol.42, no. 4,  pp. 498- 510, 1997.

[9] M. H. Heymann and F. Lin, “Discrete-Event  Control of Non-deterministic Systems”, IEEE  Trans. Auto. Control, vol.43, no.1, pp. 3-17, 1998.

[10] T. Ushio, “On the Existence of Finite-State Supervisors Under Partial Observations”, IEEE Trans. Auto. Control, vol.42, no. 11, pp.1577-1581, 1997.

[11] F. Lin and W. M. Wonham, “Supervisory Control of Timed Discrete-Event Systems under Partial Observation”, IEEE Trans. Auto. Control, vol.40, no. 3, pp. 558-562, 1995.

[12] L. Yong and W. M. Wonham, “Controllability and Observability in the State-Feedback Control of Discrete-Event Systems”, Proceedings of the 27th Conf. On Decision and Control, pp. 203-208, 1988.

[13] F. Lin and W. M. Wonham, “On Observability of Discrete-Event Systems”, Information Sciences, vol. 44, no.3, pp. 173-198, 1988.

[14] P. Kozak and W. M. Wonham, “Fully Decentralized Solutions of Supervisory Control Problems”, IEEE Trans. Auto. Control, vol.40, no. 12, pp. 2094-2097, 1995.

[15] K. Rudie and W. M. Wonham, “ Think Globally, Act Locally: Decentralized Supervisory Control”, IEEE Trans. Auto. Control, vol.37, no. 11, pp. 1692-1708, 1992.

[16] F. Lin and W. M. Wonham, “Decentralized Control and Coordination of Discrete-EventSystems with Partial Observation”, IEEE Trans. Auto. Control, vol.35, no. 12, pp.1330-1337, 1997.

[17] F. Lin and W. M. Wonham, “Decentralized Supervisory Control of Discrete-Event Systems”, Information Sciences, vol. 44, no. 3, pp. 199-224, 1988.

[18] R. Cieslak, C. Desclaux, A S. Fawaz and P. Varaiya, “Supervisory Control of Discrete-Event Processes with Partial Observations”, IEEE Trans. Auto. Control, vol.33, no. 3, pp. 249-260, 1988

[19] F. Lin, “ Diagnosability of Discrete Event Systems and Its Applications”, Discrete Event Dynamic Systems, vol. 4, no. 2, pp.197-212, 1994.

[20] M. Sampath, R. Sengupta, S. Lafortune, K. Sinnamohideen and D. Teneketzis, “Diagnosability of Discrete-Event Systems”, IEEE Trans. Auto. Control, vo. 40, no. 9, pp. 1555-1575, 1995.

[21] L. E. Pinzon, H. M. Hanisch and M. A. Jafari, “ A Comparative Study of Synthesis Methods for Discrete Event Controllers”, Rutcor Research Report 20-97, RUTCOR, Rutgers Univ., Piscataway, NJ, Sep. 1997.

[22] A. B. Brandin, “The Real-Time Supervisory Control of an Experimental Manufacturing Cell”, IEEE Transactions on Robotics and Automation, vol. 12, no. 1, pp. 1-14, 1996.